Guillermo Gálvez

CPE301 – SPRING 2018

# Design Assignment 3

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1 | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 2. | CODE OF TASK 1/A |  |  |
| 3. | SCHEMATICS |  |  |
| 4. | Flow Chart |  |  |
| 5. | SCREENSHOTS OF EACH TASK OUTPUT/BOARD SETUP |  |  |
| 6. | VIDEO LINKS OF EACH DEMO |  |  |
| 7. | Git Link |  |  |

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

LM34 Temperature sensor

Bread Board

ESP8266-01

328P Xplained Mini

Jumper wires

- DEVELOPED CODE OF TASK 1/A

- /*

- * Midterm_galveg1.c

- *

- * Created: 4/7/2018 12:23:56 PM

- * Author : Guillermo Gálvez

- */

- #include <stdlib.h>

- #include <avr/io.h>

- #include <avr/interrupt.h>

- #include <util/delay.h>

- #include <stdio.h>

- #include <stdint.h>

- #define F_CPU 8000000UL

- #define FOSC 16000000			//Frequency

- #define BAUD 9600				//Baud Rate

- #define MYUBRR FOSC/16/BAUD-1	//Automatic BAUD rate calculation

- volatile uint8_t adcValue;

- /*~1.7min timer1 Counter 65536*12 * T = ~1.7 minutes*/

- volatile uint8_t fifteenPlus = 12;	//

- /*Function Declarations*/

- void USART_Init();

- void ADC_Init();

- void outputChr(unsigned char c);

- void outputStr(char *c);

- void readTemp();

- void AT_Tx(char *t);

- //void append(unsigned char *one, unsigned char *two);

- /*Interrupt Service Routines*/

- ISR(ADC_vect)

- {

- ADCSRA	|= (1 << ADIF);	//Reset flag

- adcValue = ADCH;		//MSB 8-bits of ADC form left shift of ADLAR

- }//end ISR(ADC_vect(

- ISR(TIMER1_OVF_vect)

- {

- TIFR1 |= (1 << TOV1);	//Clr Flag

- fifteenPlus++;

- char seeTemp[8];

- float	lm34_0;		//For ASCII Temp output

- while((ADCSRA & (1 << ADIF)) == 0);			//Wait for conversion to finish

- /*Conversion to °F*/

- lm34_0 = (adcValue * 5.0 / 0x100) * 100.0;	//(ADC * 5 = 200 /256) * 100

- dtostrf(lm34_0, 5, 2, seeTemp);				//Float to char conversion

- if(fifteenPlus>=12)		//Timer

- {

- /*Build Strings for AT+ commands*/

- unsigned char CIPStart[]	= "AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n";

- unsigned char CIPSend[]		= "AT+CIPSEND=51\r\n";

- unsigned char Data[]		= "GET /update?api_key=X5GNOJ6AFIEO0XRP&field1=";

- unsigned char temp0			= "t";

- unsigned char temp1			= "\r\n";

- unsigned char CIPClose[]	= "AT+CIPCLOSE\r\n";

- _delay_ms(2000);

- outputStr(CIPStart);	//Send Start String

- _delay_ms(2000);

- outputStr(CIPSend);		//Number of Char being sent

- _delay_ms(2000);

- outputStr(Data);		//Get command sent

- outputStr(seeTemp);			//Temperature string added to end of Get command

- outputStr("\n\r");		//Enter key

- fifteenPlus = 0;		//Reset timer

- }//end if

- }//end ISR(TIMER1_OVF_vect)

- /**************************************************

- ***************************************************

- ***************************************************

- MAIN***********************************************

- */

- int main(void)

- {

- TCCR1B	|= (1 << CS12) | (1 << CS10);	//Set prescale 1024

- TIMSK1 |= (1 << TOIE1);					//enable OVF interrupt

- unsigned int oneFive = 0;

- ADC_Init();			//initialize ADC

- USART_Init();		//UART initialization

- while (1)

- {

- }//end while(1)

- }//end main

- /*USART initialization function*/

- void USART_Init()

- {

- /*Set Baud Rate*/

- UBRR0H = (MYUBRR>>8);	//Shift MSB "top" of UBRR0H	0100 0100 >> 8 -> UBRR0H 0000 0000

- UBRR0L = MYUBRR;		//UBRR0L 0100 0100

- UCSR0B |= (1 << RXEN0) | (1 << TXEN0);	 //Enable Rec and Trans

- UCSR0B |= (1 << RXCIE0);				 //Enable Rec INT

- UCSR0C |= (1 << UCSZ01) | (1 << UCSZ00); //Set frame 8-bit, 1 STP

- }//end USART_int

- /*ADC initialization function*/

- void ADC_Init()

- {

- DDRC	= 0;			//Set PORTC as input for adc

- DIDR0	= 0x01;			//Disable Digi input on ADC0

- ADMUX = 0;				//Sets Mux selection bits to 0 ADC0 used

- ADMUX |= (1 << REFS0);	//Use Vcc Ref voltage selectin 01

- ADMUX |= (1 << ADLAR);	//Left adjust ADC Reg, ADCH 8-bit Resolution

- //Enable ADC, Start Conv, Set Auto Trig Enable

- ADCSRA |= (1 << ADEN);	//Enable ADC

- ADCSRA |= (1 << ADATE); //Set ADC Auto Trig

- ADCSRA |= (1 << ADIE);	//Enable Interrupts

- ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (0 << ADPS0); //8MHz, Pre-Scale 64 = 125KHz

- ADCSRA |= (1 << ADSC);	//Start Conversion

- ADCSRB = 0;			//Free running mode

- sei();				//Enable interrupts

- }//end ADC_Init

- /*Character print function*/

- void outputChr(unsigned char c)

- {

- UDR0 = c;	//Display Char on Serial

- _delay_ms(800);

- }//end outputChr

- /*String print function*/

- void outputStr(char *c)

- {

- unsigned int i = 0;	//loop control

- while(c[i] != 0)

- outputChr(c[i++]);

- }//end outpuStr

- SCHEMATICS

Use fritzing.org  Freezes

- FLOWCHART

Setup TIMER1

Initialize ADC/USART

Wait for OVF

Take Temperature reading

If ~3.5 minutes has passed update Thingspeak through

USART/ESP8266-01

- SCREENSHOT OF (ATMEL STUDIO OUTPUT)/ (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

For video modified code to update faster changed back

https://youtu.be/Rp3zeCU4gQw

- GITHUB LINK OF THIS DA

https://github.com/galveg1/Design-Assignments/tree/master/MidT

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Guillermo Gálvez